Spacer Bars Rules:

Space bar to be considered with 25mm diameter(hardcoded) with spacing of 1m where at least two reinforcement bars come together.

Eg 1. when there is top bar and top bar extra together.

Eg2. when there are two top bars together.

Eg 3. when there is bottom bar and bottom bar extra together.

Eg 4. when there are two bottom bars together.

Where spacer bar should not considered:

Eg 1. when there are only top bar and no top bar extra together.

Eg2. when there is only top bar.

Eg 3. when there are only bottom bar and no bottom bar extra together.

Eg4. when there is only bottom bar.

Note – Spacer bar interpretation to be hardcoded and not be derived from general notes (exceptional case).

Note2- Estimator validated sheets might have missed spacer bars computations in some beams so need not match for all beams. The above logic should override in this case. 

Hardcoded Rules:

- Dia of spacer bars = 25mm

- Spacing of spacer bars = 1000mm (every 1m where two bars come together)

- Number of Spacer Bars = (Lenth of overlapping bar / spacing) +1

- Cut Length = Width of Beam – 2 * Beam Covers