STEEL BEAM ESTIMATION

Current Performance & Vision Impact  ·  Second to Sixth Sets

Deterministic model = production authority.  Claude Vision = shadow / research only.  First Set is excluded from all aggregate KPIs.

CURRENT DETERMINISTIC MODEL — SECOND TO SIXTH SETS

| Beam identification | Bar identification | Correct of detected bars | Diameter identification |
| --- | --- | --- | --- |
| 85.12% | 50.37% | 33.95% | 75.67% |

| Bar matching (vs GT) | Steel accuracy (kg-pooled) | Overall accuracy | Vision (Fifth Set only) |
| --- | --- | --- | --- |
| 17.10% | 73.59% | 60.76% | +3.02 pp shadow |

The model identifies 515 of 605 ground-truth beams (85.12%) and 2,100 of 4,169 ground-truth bar lines (50.37%). Of the 2,100 bars it does identify, 713 are a full MATCH on role, diameter and quantity (33.95%). Diameter is correct on 75.67% of those detected bars (WRONG_DIAMETER excluded). Combined steel is 109,580 kg versus 148,911 kg estimator (73.59% QA.2A steel accuracy on pooled kg). Overall accuracy is the established four-metric mean: beam identification, bar identification, correct identification of detected bars, and pooled steel accuracy.

These figures are the current production-authority result. They do not include Claude Vision.

1. Drawing-wise deterministic performance

| Drawing set | Beam ID | Bar ID | Correct of detected | Diameter ID | Bar matching | Steel | Overall |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Second Set (known) | 95.52% | 71.76% | 35.74% | 77.98% | 25.65% | 85.50% | 72.13% |
| Third Set (known) | 90.48% | 63.52% | 17.91% | 68.24% | 11.37% | 87.89% | 64.95% |
| Fourth Set (unseen) | 78.32% | 39.89% | 37.33% | 73.07% | 14.89% | 58.39% | 53.48% |
| Fifth Set (unseen) | 76.47% | 40.25% | 34.30% | 74.23% | 13.80% | 61.69% | 53.18% |
| Sixth Set (unseen) | 95.86% | 61.45% | 38.87% | 81.63% | 23.89% | 96.54% | 73.18% |
| ALL FIVE SETS | 85.12% | 50.37% | 33.95% | 75.67% | 17.10% | 73.59% | 60.76% |

Beam ID = detected beams / GT beams.  Bar ID = detected bar lines / GT bar lines.  Correct of detected = MATCH / detected (QA.2A bar_accuracy).  Bar matching = MATCH / GT bars.  Diameter ID = (detected − WRONG_DIAMETER) / detected.  Steel = QA.2A 100 − |model−estimator| / estimator; five-set row is kg-pooled, not an average of %.  Overall = mean of Beam ID, Bar ID, Correct of detected, and Steel.  Known = Second–Third (development).  Unseen = Fourth–Sixth (generalization).

2. What the five questions mean in numbers

| Question | KPI | Five-set result | Raw counts |
| --- | --- | --- | --- |
| What % of beams does the model identify? | Beam identification | 85.12% | 515 / 605 GT beams |
| What % of bars does the model identify? | Bar identification | 50.37% | 2,100 / 4,169 GT bar lines |
| Of bars identified, what % are correct? | Correct of detected bars | 33.95% | 713 MATCH / 2,100 detected |
| How accurately are diameters identified? | Diameter identification | 75.67% | (2,100 − 511 WRONG_DIA) / 2,100 |
| How close is steel quantity to the estimator? | Steel accuracy | 73.59% | 109,580 / 148,911 kg |

3. Known (Second–Third) vs unseen (Fourth–Sixth)

| Metric | Second–Third known | Fourth–Sixth unseen | All five sets |
| --- | --- | --- | --- |
| Beam identification | 93.08% (121/130) | 82.95% (394/475) | 85.12% |
| Bar identification | 67.25% (573/852) | 46.04% (1,527/3,317) | 50.37% |
| Correct of detected bars | 26.53% (152/573) | 36.74% (561/1,527) | 33.95% |
| Diameter identification | 73.00% | 76.69% | 75.67% |
| Bar matching vs GT | 17.84% (152/852) | 16.91% (561/3,317) | 17.10% |
| Steel accuracy (QA.2A set-mean) | 86.70% | 72.21% | 78.00%* |
| Steel accuracy (kg-pooled) | 98.37%† | 68.23% | 73.59% |
| Overall (pooled steel in five-set) | 68.39%‡ | 59.49%‡ | 60.76% |

* QA.2A dashboard steel for multiple sets is the unweighted mean of per-set steel %.  † Known kg-pooled steel is inflated by Second-Set over-estimate cancelling Third-Set under-estimate — do not read 98% as known-set quality.  ‡ Group overall uses QA.2A set-mean steel.  Five-set overall uses kg-pooled steel.

4. Diameter identification (detected bar lines)

QA.2A’s published field diameter_accuracy_pct is an alias of bar matching accuracy (MATCH / detected) and is not used here. Diameter identification below is counted from Bar Matching rows: a detected bar is diameter-correct unless status is WRONG_DIAMETER. GT diameter is the estimator line diameter. Ø32 has only 8 detected lines — percentage unstable.

| Diameter | GT bar lines | Detected | MATCH | WRONG_DIA | Diameter ID | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Ø8 | 358 | 159 | 37 | 3 | 98.11% | Usually right when found; many missing |
| Ø10 | 714 | 235 | 72 | 10 | 95.74% | Same pattern — detection is the gap |
| Ø12 | 573 | 224 | 64 | 31 | 86.16% |  |
| Ø16 | 588 | 404 | 155 | 140 | 65.35% | Frequent diameter swaps |
| Ø20 | 784 | 431 | 151 | 186 | 56.84% | Weakest major diameter |
| Ø25 | 1,097 | 639 | 231 | 136 | 78.72% | Includes many spacer lines |
| Ø32 | 26 | 8 | 3 | 5 | 37.50% | Low volume — unstable |
| TOTAL scored Ø8–Ø32 | 4,140 | 2,100 | 713 | 511 | 75.67% | Five-set headline |

5. Diameter-wise steel quantity — not the same as diameter identification

Quantity ratio = automated kg / estimated kg × 100. It is not accuracy. A ratio above 100% is an overestimate. Ø16 is 143% of estimator kg while Ø10 is 50%.

| Diameter | Estimated kg | Automated kg | Difference kg | Abs % diff | Quantity ratio |
| --- | --- | --- | --- | --- | --- |
| Ø8 | 6,515 | 4,389 | −2,126 | 32.6% | 67% |
| Ø10 | 24,555 | 12,236 | −12,319 | 50.2% | 50% |
| Ø12 | 22,253 | 11,670 | −10,582 | 47.6% | 52% |
| Ø16 | 11,598 | 16,564 | +4,965 | 42.8% | 143% |
| Ø20 | 35,888 | 29,320 | −6,568 | 18.3% | 82% |
| Ø25 | 45,294 | 32,965 | −12,328 | 27.2% | 73% |
| Ø32 | 2,808 | 2,436 | −372 | 13.2% | 87% |
| TOTAL | 148,911 | 109,580 | −39,330 | 26.4% | 74% |

6. Error profile — Second to Sixth Sets

| Error category | Second | Third | Fourth | Fifth | Sixth | Five-set |
| --- | --- | --- | --- | --- | --- | --- |
| Missing bars | 109 | 170 | 565 | 870 | 355 | 2,069 |
| Wrong quantity | 81 | 114 | 103 | 166 | 176 | 640 |
| Wrong diameter | 61 | 94 | 101 | 151 | 104 | 511 |
| Extra bars | 74 | 94 | 51 | 123 | 81 | 423 |
| Wrong role | 30 | 23 | 20 | 45 | 40 | 158 |
| Beam missing | 3 | 6 | 31 | 44 | 6 | 90 |
| Beam mismatch | 0 | 4 | 6 | 0 | 4 | 14 |
| Steel difference | 1 | 1 | 1 | 1 | 1 | 5 |
| Total classified errors | 359 | 506 | 878 | 1,400 | 767 | 3,910 |

Missing bars dominate unseen sets. Diameter and quantity errors remain material even when a bar is detected. This is why 50% bar identification does not become 50% correct steel.

7. Claude Vision — experimental, Fifth Set only

Engineering-level Vision-assisted recompute has currently been demonstrated on the Fifth Set. No valid engineering recompute exists for Second, Third, Fourth or Sixth Sets. A five-set Vision KPI is therefore not calculated. P2.5.8 is the authoritative Vision engineering result. P2.5.9–P2.5.11 are later safety/arbitration experiments and are not production Vision accuracy.

| Fifth Set steel | Deterministic (production) | Vision-assisted (P2.5.8 shadow) | Estimator |
| --- | --- | --- | --- |
| Steel kg | 36,271.794 | 38,049.747 | 58,796.332 |
| Steel accuracy | 61.69% | 64.71% | — |
| Absolute error | 38.31% | 35.29% | — |

| Accuracy lift | Error reduction | Stirrup accuracy | Beams ↑ / = / ↓ |
| --- | --- | --- | --- |
| +3.02 pp | 7.88% | 35.44% → 44.35% | 21 / 150 / 14 |

P2.5.8 replayed frozen P2.5.7 Claude results (0 new API calls) onto a sandbox copy of the reinforcement model. Practical value was OCR-difficult stirrups and slash spacing such as 100/125/100 that SI.1 truncated to the first token. Promoted fields: diameter 29, legs 29, spacing 35, role 0. Production mutation = 0; steel / BBS / Excel production difference = 0.

P2.5.8 DECISION · NEGATIVE · NOT PRODUCTION-SAFE

Do not present 64.71% as current production accuracy. Fourteen beams worsened. Unrestricted new-stirrup inserts were rejected. P2.5.9–P2.5.11 gate those inserts (UNKNOWN-only, then evidence HOLD) and drop the lift to about +0.3 pp with zero worsenings.

8. Current workflow value — model-assisted, not autonomous

DXF  →  beam discovery  →  reinforcement interpretation  →  stirrup interpretation  →  steel calculation  →  BBS  →  Excel output  →  estimator verification.

Already automated: beam discovery (~85% of GT beams), annotation parsing, stirrup parsing, IS 456 steel calculation, BBS and Excel. Still required: finding the ~15% missed beams, the ~50% missed bars, and correcting ~66% of detected bars that are not a full MATCH. Positioning: model-assisted estimation / estimator first-pass support.

9. Indicative estimator time-saving potential

No estimator time-and-motion study, manual-vs-model hours, or productivity log exists in the project. Accuracy percentages are not time saved. 50% bar identification is not 50% time saved, because missed bars and incorrect detected bars still require drawing reading.

| If used today as a first-pass worksheet | Indicative range | Basis (not a measurement) |
| --- | --- | --- |
| Beam take-off / marking | Partial save | 85% of GT beams already listed |
| Bar schedule compilation | Limited save | Half of GT bars missing; 2 in 3 detected bars need edit |
| Steel calc / BBS / Excel typing | High save | Fully automated for whatever the model produced |
| Verification / red-line | Still required | Unseen-set steel 58–62% except Sixth Set |
| Overall estimation time | 20–35% | Indicative only — requires time-and-motion study |

Indicative time-saving potential today: 20–35% of manual estimation time, if the Excel is used as a first pass and the estimator still verifies every beam. This is not reliably measurable until a timed estimator study is run. Computer pipeline runtime (e.g. Fifth Set ~2.8 h) is model processing, not estimator saving.

10. Current status

| Item | Value |
| --- | --- |
| Beam identification | 85.12% |
| Bar identification | 50.37% |
| Correct identification of detected bars | 33.95% |
| Diameter identification accuracy | 75.67% |
| Bar matching accuracy (MATCH / GT) | 17.10% |
| Deterministic steel accuracy (kg-pooled) | 73.59% |
| Overall accuracy | 60.76% |
| Vision-assisted engineering improvement | Fifth Set only: 61.69% → 64.71% (+3.02 pp, shadow) |
| Indicative estimator time saving | 20–35% · not yet reliably measurable |
| Current usage | Model-assisted estimation / estimator first-pass support |
| Production authority | Deterministic model |
| Claude Vision | Controlled shadow / research · P2.5.8 NEGATIVE for unrestricted promotion |

Claude Vision has demonstrated a real engineering lift on difficult stirrup/OCR cases, but unrestricted promotion worsened 14 Fifth-Set beams. P2.5.9–P2.5.11 are making recovery engineering-safe rather than maximising aggregate accuracy. Production readiness is not claimed.

11. Methodology, sources, limitations

Second–Third: Version 9 QA.2B.1 GroundTruth_Benchmark_Report.xlsx (known development sets).  Fourth–Sixth: Version 10 QA.3.0 Generalization_Benchmark_Report.xlsx and per-set benchmark_result.json (unseen production runs qa2_*_20260806).  Counts are Excel Bar Detection / Bar Matching integers, not rounded percentages from the old six-set PDF.  Steel kg from QA.2A metric8.  Diameter kg from Diameter Steel Quantity sheets.  Vision from P2.5.8_STATUS.md.  First Set excluded (GT 18 beams / 85 bars / 1,424 kg).  Limitations: no estimator time study; diameter ID is status-based (PARTIAL_MATCH residual exists); Ø1/Ø2 rows in quantity sheets are parser artefacts and were omitted; Vision engineering recompute exists only for Fifth Set.