For Steel Estimation of Beams

Aim – To calculate steel quantity in KG/Metric Ton w.r.t Diameter of the steel bars and then total.

Estimators will consider 3 drawings in parallel in this Estimation Process

- General Notes give per project –

File Name - SE-1000RO-SH-01&SH-02(GENERAL NOTES).dxf

Note1:

- Read and Extract from General Note Drawing Sheet provided for the project (SE-1000RO-SH-01&SH-02(GENERAL NOTES).dxf. Analyze this file completely.

- If anywhere it is mentioned as General Note, it means (SE-1000RO-SH-01&SH-02(GENERAL NOTES).dxf for this project.

- Sheet Name - SE-GN-100-SH-02 – This sheet is more important and used for majority of computations.

- Development Length & Anchorage table to be considered and extracted from General Note Drawing of the Project as per Grade of Steel.

Sheet Name - SE-GN-100-SH-02

2. Grade of Steel to be considered from Table2 of General Note

3. Grade of Concrete to be considered from Table2 of General Note

4. Cover of Concrete to be considered from Table2 of General Note

5. Thickness of Slab to be considered from Framing plan of the beams from drawing plan.

6. Space bar details to be considered as 25mm diameter with spacing of 1m where at least 2 reinforcement bars come together.

- Framing Plan and GFC reinforcement plan /drawings

File Name - SteelBeam_Galera_STR&OHT_Top.dxf

Framing Plan/ Drawing

    - (To measure the dimensions of the beam)

    - To measure and extract Clear Length of Beam ( Center Line Method)

    - To measure and extract Width of the Beam

    - To measure and extract Depth of the Beam

- Reinforcement plan /drawings

GFC Reinforcement Drawing

    - To read and extract number of the bars

    - To read and extract diameter of the bars

    - To read and extract spacing of the bars

    - Different types of Bars to be read and extracted according to drawings

      - Top Bar

      - Top Bar – extra

      - Bottom Bar

      - Bottom Bar – extra

      - Stirrups

      - SFR – Side Face Reinforcement (Usually only for Curved Beam)

      - Space Bar

Reading GFC Drawings:

Type1 (Top Bar, Top Bar extra, Bottom Bar, Bottom Bar extra and SFR)

- 2Y-16 = 2 Number of Bars with Diameter 16mm

- 2Y-20 = 2 Number of Bars with Diameter 16mm

- 4Y-8 = 4 Number of Bars with Diameter 8mm

Type2 (Stirrups with uniform spacing)

- 2L-Y10@100C/C = 2-Legged stirrups with diameter 10mm placed 100mm all along the beam.

- 2L-Y8@150C/C = 2-Legged stirrups with diameter 8mm placed 150mm all along the beam.

Type3 (Stirrups with varying spacing)

- 2L-Y8@100/200/100C/C = 2-Legged stirrups with diameter 8mm placed at 2 different spacings by dividing the beam into Number of  parts along the length of beam mentioned in the drawings (In this case its 3 parts – 100/200/100C/C).

  - First part 100mm spacing

  - Middle part 200mm spacing

  - Last part 100mm spacing

  - First part+Last part usually has same spacings so calculated together.

Type4 (Dimensions mentioned on beam reinforcement

  - In Beam5(B5) - 2150 = Clear length of Top bar – extra

  - In Beam5(B5) – 500 = Clear length of Bottom bar – extra to be deducted(subtracted) from Clear length of Bottom bar so that we can arrive at Clear length of Bottom bar- extra.

  - In Beam4(B4) - 2150 = Clear length of Top bar – extra

  - In Beam4(B4) – 500 = Clear length of Bottom bar – extra to be deducted(subtracted) from Clear length of Bottom bar so that we can arrive at Clear length of Bottom bar- extra.

- Excel Estimator Output Sheet – This is the steel beam estimation output sheet computed by the estimators manually. This must be automated.

File Name - Galera_SteelBeamEst_SHR&OHT_TopFramingPan.xlsx

- The model must learn the pattern to compute estimation of steel beams like the format provide and table as well.

- This sheet won’t be provided every time. Consider this to only understand how to read and interpret beam drawings to prepare accurate estimates.

| Note2:(To understand what abbreviations means in estimator excel sheet) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No/Dia | No - Number of Beams |  |  |  | Dia - Diameter of the Bar |  |
| L/Spcng (m) | L - Clear Length of the Bar in m |  |  | Spcg - Spacing of bar in m |  |
| B(m)/No. (m) | B - Width of the Beam |  |  | No. - Number of Bars |  |
| D/Dvlp L (m) | D - Depth of Beam |  |  | Devl L - Development Length of the bar |  |

Formulae:

- Clear Length of the Bar – To be measured directly from Framing Plan

- Width and depth of Beam - To be extracted directly from Framing Plan

- Development Length of Bar – From Table2 in General Note Sheet 2. Sheet Name - SE-GN-100-SH-02. Diameter specific.

- Cutting Length = Clear Length + Development Length + Bend Correction

- Bend Correction = (2 * Dia of Bar in m) /1000

- Total Length = Cutting Length * Number of Bars

- Steel in KG = Total Length * Unit Weight of Bars

- Unit Weight of Bar depends on Diameter of the Bar (This has to be considered from the excel sheet directly).

Spacer Bars Rules:

Space bar to be considered as 25mm diameter(hardcoded) with spacing of 1m where at least two reinforcement bars come together.

Eg 1. when there is top bar and top bar extra together.

Eg2. when there are two top bars together.

Eg 3. when there is bottom bar and bottom bar extra together.

Eg 4. when there are two bottom bars together.

Where spacer bar should not considered:

Eg 1. when there are only top bar and no top bar extra together.

Eg2. when there is only top bar.

Eg 3. when there are only bottom bar and no bottom bar extra together.

Eg4. when there is only bottom bar.

Note – Spacer bar interpretation to be hardcoded and not be derived from general notes (exceptional case).

Note2- Estimator validated sheets might have missed spacer bars computations in some beams so need not match for all beams. The above logic should override. 

Hardcoded Rules:

- Dia of spacer bars = 25mm

- Spacing of spacer bars = 1000mm (every 1m where two bars come together)

- Number of Spacer Bars = (Lenth of overlapping bar / spacing) +1

- Cut Length = Width of Beam – 2 * Beam Covers