AI DRIVEN ESTIMATION AUTOMATION
Overview Report

# 1. Construction Estimation Scope

Based on discussions with the Estimation Team, construction estimation covers the following major material and work categories. Concrete, Steel and Earthwork are identified as major aspects.

| SL No | Material / Work Category | Importance | Current AI Coverage / Scope |
| --- | --- | --- | --- |
| 1 | Concrete | High Priority | Beam & Slab currently developed |
| 2 | Steel | High Priority | Beam currently developed |
| 3 | Blockwork | Standard Scope | Wall / block quantities |
| 4 | Tiles & Granite | Standard Scope | Floor / wall finishes |
| 5 | Plastering | Standard Scope | Internal / external surfaces |
| 6 | Painting | Standard Scope | Paintable surfaces |
| 7 | Waterproofing | Standard Scope | Wet areas / terraces |
| 8 | Doors & Windows | Standard Scope | Openings / glazing |
| 9 | Earthwork | Standard Scope | Excavation / filling quantities |

# 2. Major Structural Components

The structural estimation framework is organized around six major components. The current development covers Concrete for Beam and Slab and Steel for Beam.

| Component | Concrete | Steel | Current Position |
| --- | --- | --- | --- |
| Beam | Developed | Developed | Current development focus |
| Slab | Developed | Future | Major structural component |
| Column | Future | Future | Structural member |
| Foundation | Future | Future | Foundation estimation |
| Shear Wall | Future | Future | Vertical structural wall |
| Retaining Wall | Future | Future | Earth-support structure |

# 3. Construction Estimation Involves

INPUT → INTERPRET & MEASURE → CALCULATE → VALIDATE → OUTPUT

INPUT: Drawings, schedules, notes and revisions.

INTERPRET & MEASURE: Geometry, dimensions, areas, lengths and counts.

CALCULATE: Engineering rules, formulas and project practices.

VALIDATE: Quantity checks, anomalies and estimator review.

OUTPUT: Quantity take-off, BOQ-ready data and Excel outputs.

AI reduces repetitive interpretation effort, while engineering logic remains central to quantity calculation.

# 4. Current AI Development — What Has Been Built

| CONCRETE — BEAM & SLAB | STEEL — BEAM |
| --- | --- |
| • Beam concrete | • Reinforcement drawing understanding |
| • Beam shuttering | • Beam matching / ownership |
| • Slab concrete | • Reinforcement parsing |
| • Slab shuttering | • Steel quantity + BBS |
| • Deterministic calculation | • Hybrid CAD + Vision |
| • LLM assistance for ambiguous inputs | • Deterministic engineering authority |

Common foundation: reusable drawing intelligence → material/component-specific estimation engines

# 5. Concrete Beam & Slab Estimation

The Concrete Beam & Slab Estimation model processes structural DXF framing drawings, identifies structural members, performs engineering calculations and produces estimator-oriented outputs.

DRAWING → ENGINEERING → OUTPUT

| DRAWING | ENGINEERING | OUTPUT |
| --- | --- | --- |
| DXF framing drawing Member detection Geometry / dimensions | Concrete volume Shuttering area Rules & calculations | Excel quantity report Review sheet Flagged items |

Standard drawings can be processed deterministically.

LLM assistance is used only when information such as slab thickness, tags, beam dimensions or bounded regions is missing / ambiguous.

Estimation Team validation demonstrated meaningful productivity improvement; quantity accuracy still requires further improvement.

Concrete Beam & Slab Estimation Model Link: http://13.201.15.89

# 6. Concrete POC — Estimation Team Validation: Accuracy & Time Saving

| Metric | Result |
| --- | --- |
| Slab Concrete | 69% |
| Slab Shuttering | 73% |
| Beam Concrete | 78% |
| Beam Shuttering | 81% |
| Slab Productivity | 29% time saving |
| Beam Productivity | 67% time saving |

Estimation Team conclusion: processing effort is reduced, but current quantities still require accuracy improvement.

# 7. Steel Beam Estimation — Hybrid AI Architecture

The Steel Beam model uses Hybrid AI architecture that combines deterministic CAD / engineering information with AI Vision for semantic reinforcement interpretation. Final quantity calculation remains under deterministic engineering control.

| DETERMINISTIC CAD / ENGINEERING | AI VISION | HYBRID RESOLUTION |
| --- | --- | --- |
| Geometry Beam model Spans / dimensions Cover Engineering rules | Reinforcement groups Bar count / diameter Top / Bottom / Main / Extra Support scope Visual stirrups | Combine semantic interpretation with deterministic facts and project rules before quantity computation. |

VISION: “What reinforcement exists?”  → Hybrid Resolution → ENGINEERING: “How is it quantified?”

# 8. Steel Beam Model — Engineering Intelligence Stack

| Capability | Coverage |
| --- | --- |
| Drawing Architecture | Identity • drawing sets • beam index • versioning |
| Geometry Intelligence | Regions • sketches • text • leaders • blocks |
| Detail & Match | Multi-view recognition • context • candidate ranking • confidence |
| Ownership | Text / leader / bar / stirrup / shared ownership |
| Reinforcement Parsing | Longitudinal bars • stirrups • side-face • laps • hooks • curtailment |
| Quantity & Output | Cut lengths • unit weights • steel quantity • BBS • Excel |

# 9. End-to-End Steel Estimation Workflow

Upload drawing package

Understand drawing

Extract geometry & context

Identify components / material regions

Route to estimation engine

AI / Vision where required

Hybrid resolution

Deterministic quantity calculation

Validation & review

BOQ / BBS / Excel

# 10. Steel Beam — Internal Accuracy Testing

The following results represent current internal model testing. They are not final acceptance results and remain subject to Estimation Team validation.

| Diameter (mm) | Automated (MT) | Estimated (MT) | Difference | Error Scale | % Error | % Accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| Y8 | 0.55 | 0.43 | 0.12 | Overestimate | 27.57% | 72.43% |
| Y10 | 0.91 | 1.18 | -0.27 | Underestimate | -23.12% | 76.88% |
| Y12 | 2.31 | 2.78 | -0.47 | Underestimate | -16.98% | 83.02% |
| Y16 | 3.51 | 2.57 | 0.94 | Overestimate | 36.49% | 63.51% |
| Y20 | 3.00 | 2.33 | 0.67 | Overestimate | 28.54% | 71.46% |
| Y25 | 1.31 | 1.12 | 0.19 | Overestimate | 17.25% | 82.75% |
| TOTAL | 11.58 | 10.41 | 1.17 | Overestimate | 11.24% | 88.76% |

Internal testing result: 70–80% overall accuracy • Final performance subject to Estimation Team validation.

Note: the diameter-wise table shows an aggregate of 88.76% accuracy based on the listed automated and estimated steel quantities. The presentation summarizes the broader current internal testing outcome as 70–80% overall accuracy. Both are internal testing indicators until validated by the Estimation Team.

Steel Beam Estimation Model Link: http://13.127.104.99/

# 11. Complete AI Estimation Platform — Overall Direction

The current Concrete and Steel work is positioned as the foundation for a broader multi-material and multi-component AI Estimation platform.

| Platform Layer | Purpose |
| --- | --- |
| 1. Input | DXF / CAD • PDF • Image • schedules |
| 2. Drawing Intelligence | Parsing • OCR / text • symbols • regions • spatial context |
| 3. Engineering Geometry | Dimensions • spans • areas • lengths • member geometry |
| 4. Material / Component Engines | Concrete • Steel • Earthwork • Blockwork • Finishes • Openings |
| 5. Quantity Engine | Engineering rules • formulas • units • estimator practices |
| 6. Validation | Confidence • anomalies • reconciliation • human review |
| 7. Output | BOQ • BBS • Excel • summaries • audit trail |

# 12. Expected Business Value

Reduce repetitive manual drawing interpretation and quantity take-off effort.

Improve estimation productivity while retaining engineering review where needed.

Standardize quantity calculation logic and estimator practices.

Improve traceability from estimated quantity to drawing evidence and engineering rules.

Create a scalable foundation for multiple materials and structural components.

Support faster handling of drawing revisions and repeated project estimation.

Build structured data and validation feedback for continuous AI improvement.

# 13. Core Design Principles

Engineering correctness takes priority over model complexity.

AI interprets difficult drawing semantics; deterministic engineering logic calculates quantities wherever possible.

AI outputs are reconciled with geometry, engineering rules and estimator practices.

Low-confidence or anomalous cases are surfaced for review rather than silently accepted.

The architecture is modular and reusable across materials and structural components.

Validation of varied and unseen drawings is essential before production reliance.

# 14. Model Architecture

DRAWING PACKAGE
↓
DRAWING INTELLIGENCE
↓
MATERIAL + STRUCTURAL COMPONENT INTELLIGENCE
↓
AI / VISION WHERE REQUIRED
↓
HYBRID RESOLUTION
↓
DETERMINISTIC ENGINEERING CALCULATION
↓
VALIDATED QUANTITIES → BOQ / BBS / EXCEL

The long-term objective is a unified AI-assisted construction estimation platform covering the major material categories and structural components identified with the Estimation Team. The current Concrete Beam/Slab and Steel Beam work provides the foundation for this broader estimation automation capability.